Project 1:  FMCW Radar Target Detection: CFAR vs Deep Learning on Range–Doppler Maps

Overview:
FMCW radars estimate target range and radial velocity by processing received chirps into a Range–Doppler (RD) map. Classical detection uses CFAR thresholding, which can degrade under low SNR, non-homogeneous clutter, multipath, and multi-target masking. This project builds a complete FMCW processing pipeline and integrates Machine Learning / Deep Learning to detect targets from RD maps, then benchmarks ML against CFAR using rigorous detection metrics.

Main Objective:

Implement and evaluate an FMCW radar detection system, comparing CA-CFAR / OS-CFAR to a deep-learning detector operating on RD maps.

Key Tasks / Methodology:

- Data generation (simulation with labels): Generate FMCW frames with 0–5 targets/frame; random range/velocity/amplitude; SNR sweep (e.g., −10 to +20 dB); add clutter (near-zero Doppler energy) and optional multipath/non-idealities (frequency offset/phase noise). Labels are target bin locations.

- Radar processing: Dechirp → Range FFT → Doppler FFT → RD map (log magnitude).

- Baselines: Implement CA-CFAR (mandatory) and OS-CFAR (recommended), with tunable guard/training cells and desired Pfa.

- ML/DL module: Train a CNN/U-Net to output a target heatmap/segmentation mask from RD maps; post-process with peak finding + non-maximum suppression.

- Benchmarking: Compare CFAR vs DL on identical test scenarios.

Evaluation Metrics:

- Pd vs Pfa, ROC curves (threshold sweep)

- Pd at fixed Pfa

- Localization error in (range bin, Doppler bin) and optionally meters & m/s

- Runtime per frame + model size

Ablation Study (experiments):

SNR sweep; clutter strength; number of targets; close-target separation; RD resolution (bandwidth, chirps); CFAR parameter sensitivity; DL model size and input type (magnitude vs I/Q channels).

Expected Deliverables:

- FMCW simulator + RD pipeline (Python)

- CA-CFAR + OS-CFAR implementations

- Labeled RD dataset generator (train/val/test)

- Trained DL detector + inference scripts

- Benchmark report (ROC, Pd/Pfa tables, localization, runtime) + plots

- Reproducible GitHub repo (configs, requirements, instructions)

- Optional demo (Streamlit/Gradio) visualizing RD map + detections

Tools / Stack:

Python 3.10+, NumPy/SciPy, Matplotlib, PyTorch, (optional) scikit-image/OpenCV, (optional) Numba, (optional) Streamlit/Gradio.

References (starter):

Richards (2014), Skolnik (2001), Principles of Modern Radar (2010), Rohling (1983) CFAR.

Team Size: 1–2 students (2 ideal).